METEOROLOGICAL INPUTS Ta, RH, u, Fsd, Fld, SWC + measured Rn (for G and ΔQc) half-hourly tower record (Table 1) · observed fluxes for calibration and evaluation (EddyPro v7.0.9, PyFluxPro v3.4)

↓

INITIALIZE SURFACE TEMPERATURE Ts = Ta + 3 K first guess for the iterative solver

↓

NET RADIATION Rn = (1 − α)Fsd + εFld − εσTs4 α(t) = αnoon + Δα·[1 − cos(π¦t − 12¦/6)], αnoon = 0.098, Δα = 0.0065 gray body: ε = 0.97 for the absorbed and the emitted longwave radiation

↓

ELECTRICITY GENERATION Eelec = ηFsd, η = 0.11 (Fsd > 50 W m−2) energy exported off-site as electricity, removed from the local heat budget

↓

AERODYNAMIC RESISTANCE ra ra,n = [ln((z − d)/z0m)·ln((z − d)/z0h)]/(κ2u) ra = ra,n · fseas · fstab · fRH · fm, fm = 1.57 fseas: 1.00 May–Sep, 1.02 Apr and Oct, 1.05 Dec–Feb, 1.08 Mar and Nov

↓

CANOPY RESISTANCE rc (JARVIS–STEWART) rc = rc,base · fseas(rc) · fTa · fRH(rc) · fSWC · fVPD · fFsd, 30 ≤ rc ≤ 800 s m−1 rc,base (full record): MAM 580, JJA 596, SON 591, DJF 649 s m−1 fseas(rc): 0.85 Jun–Aug, 1.20 Sep, 1.10 May and Oct, 0.90 Mar and Nov, 1.00 Apr and Dec–Feb

↓

GROUND HEAT & FOUNDATION STORAGE Gsurface = Gplate + Cs(SWC)·Δz·(dTsoil/dt), Δz = 0.10 m Cs(SWC) = (1 − φ)Cm + SWC·Cw + (φ − SWC)Ca, φ = 0.44 G = g1Rn + g2(dRn/dt) + g3 (hysteresis form, per season) ΔQc = c1Rn + c2(dRn/dt) + c3, c1 = 0.0430, c2 = 131 s, c3 = −4.3 W m−2 Rn and dRn/dt measured · G fitted to Gsurface · ΔQc: concrete-pile heat storage (OHM)

↓

TURBULENT HEAT FLUXES H = ρcp(Ts − Ta)/ra LE = ρLv[qs(Ts) − qa]/(ra + rc) rc is bypassed (ra only) when qs(Ts) < qa (dew or condensation)

↓

SURFACE-TEMPERATURE SOLVER (NEWTON–RAPHSON) Res(Ts) = Rn − H − LE − G − Eelec − ΔQc, Ts ← Ts − Res/(dRes/dTs) with the updated Ts, repeat from NET RADIATION until ¦Res¦ < 0.1 W m−2 (at most 50 iterations)

↓

OUTPUTS Rn = H + LE + G + Eelec + ΔQc (solver solution) β = H/LE diurnal residual correction of H and LE, added after the solution: ΔF = aF sin θ + bF cos θ + cF, θ = (t − 12)π/6 F = H, LE; reported flux F + ΔF five energy terms and the Bowen ratio at every half-hourly step

↓

CALIBRATION & VALIDATION season-stratified five-fold cross-validation (rc,base and diurnal correction refitted in each fold) fixed once: αnoon, Δα, g1–g3, fm (full record); c1–c3 (pile temperature record) benchmarked against principal-component regression (PCR) and a convolutional neural network (CNN) independent hold-out test: training 1 May 2024 to 30 April 2025, test 1 May 2025 to 1 February 2026 R2 (cross-validation / hold-out): H 0.90 / 0.91 · LE 0.64 / 0.65 · G 0.81 / 0.81